Roofing Types and Differences

There are 3 main types of roofing: Metal, Shingled, and Flat. Generally speaking, shingles provide the most cost effective roofing when factoring in price, warranty, ease of install and repair. Flat roofs are meant only for roofs that have a pitch of 3 or less, with a 0 pitch requiring a TPO flat roof. Metal roofing is a broad term that can cover anything from cheap tin to expensive high grade steel.

Metal Roofing

Metal roofing usually comes in 3 different materials; tin, aluminum, and steel. There are copper and zinc roofs as well, but the majority of customers don’t know of their existence. Copper roofing can be rated for 100 years, but it is a soft metal. Zinc is largely the same, boasting good corrosion resistance but softer than most metal roofing.

- Steel- The highest quality and the most expensive of the metal roofing types. Steel roofing can be further divided by gauge and whether the steel is galvanized or not. The biggest benefits of a steel roof are twofold. First, they are usually rated for 40-70 years, with some of the higher end roofs being rated for 70+. Second, steel roofing doesn’t usually care about wind speed and they have the highest impact rating amongst roofs.

- Aluminum- Aluminum lies between tin and steel in quality, and has the highest variance between products. Some aluminum roofs might be rated only for 10-20 years while the higher end roofs can be rated from 40 to 70 years. Aluminum’s main appeal over other metal roofing is that it can handle saltwater better than most of its counterparts.

- Tin- Tin is the lowest quality metal roofing that can be bought. Low quality tin roofs are just that, a thin piece of tin and not much more. Most modern tin roofs are actually a thin piece of steel or aluminum coated in tin. They can be rated anywhere from 10-50 years depending on the quality.

How to Convince a Customer Off of Metal Roofing?

- Cost - The initial price of metal roofing is much higher than a shingle equivalent. The huge sticker price attached to the product is often enough to dissuade people from going down that avenue.

- Return on Investment/Insurance Issues - Many people are attracted to the idea of a metal roof because they believe that it can last forever. Most insurance carriers will not cover a roof over 30 years, and if they do, often times the rate can be cripplingly expensive. Point out that while it may seem a good investment, if they are forced to change roofs 20-30 years in then they did not get their money’s worth.

- Warranty and Protection - Shingles historically have not offered the high end warranty coverage that metal roofs did. That is not the case anymore. Highlight to them that the best warranty a metal roof gives you is its wind warranty, which can now be reached at a much lower price point through a Timberline shingle that has the same coverage.

- Ask the Question - If a client says they want to look at metal roofing, it can be as easy as this, “Yes of course, we do both metal and shingle roofing. Now let me ask you. If I were able to find you a roof that has the exact same or better warranties as a steel roof at a quarter of the cost, would that interest you?”

Flat Roofing

Flat roofing refers to any roofing done on a roof that has less than a 3 pitch. There are two types of flat roof. There is a “flat roof” where the roof is mostly flat, but has a slight pitch. Then there are truly flat roofs that have what is called a “0 pitch”. These roofs have no pitch at all and sit dead flat. Roofs that are completely flat require a different product and more in depth process to secure them than a roof that has a slight pitch to it.

Pitch at 1-3

If the roof has a slight pitch, it can get what is sometimes referred to as a “rubber roof” or a compressed roof. The product we will be selling for the roofing of this slope is called “GAF Liberty”. Liberty is a self adhering modified bitumen roll roofing system (bitumen is a modified asphalt material enhanced with fiberglass and other materials to add strength and flexibility). Effectively, installers will push out each roll of roofing and seam them together into a cohesive barrier. If someone asks whether it is similar to a rubber roof or other flat roofing, the answer is yes, though it is NOT meant for 0 pitch.

Pitch at 0

If the roof has no pitch at all, the only way we can warranty it is with a product called TPO (Thermoplastic Polyolefin). TPO is one of the highest quality ways to do a flat roof. Oftentimes customers will be under the impression that a simple rolled roof like Liberty or a quick seal coat will be able to do their roof. TPO is in fact, one of the only materials actually meant for a 0 pitch roof. TPO is superheated to roughly 400-600 degrees then applied one roll at a time, effectively forming a seamless membrane over the entire roof. TPO installs also come with ISO Tapered Boards, a board that is custom set to provide proper grading to the roof. These roofs also get a waterproof latex membrane applied underneath called a “Red Membrane”. They are labor intensive and the material is expensive, but it is the right choice to do it correctly.